LIST OF CONSUMABLE MEDICAL SUPPLIES

FOR CLINICAL DEPARTMENTS**

1. List of Consumable Supplies for the Department of Anesthesiology and Intensive Care

| Item | Sizes / Specifications | Technical Requirements |
| --- | --- | --- |
| Endotracheal tubes with cuff | 3.0–9.0 mm | High-volume/low-pressure cuff, depth markings |
| Endotracheal tubes without cuff | 2.5–6.0 mm | For pediatric/neonatal patients |
| Oropharyngeal airways (Guedel) | 40–110 mm | Sizes 000–5, color-coded |
| HMEF breathing filters | Adult/Pediatric | Bacterial/viral filtration ≥ 99.99% |
| Breathing circuits | 1.3–1.8 m | Corrugated, antistatic |
| Closed suction systems | 24–72 h | PEEP preservation, sterile |
| Spinal anesthesia needles | 22G–27G | Quincke / Pencil-point |
| Epidural anesthesia needles | 16G–18G | Tuohy type |
| Epidural catheters | Universal | Radiopaque |
| Combined spinal–epidural kits | Adult/Pediatric | Complete sterile set |
| Guide needles | 18G–22G | Ultrasound-guided |
| Infusion pump extension lines | 1.5–3 m | Luer-lock |
| ECG electrodes | Disposable | Solid gel |
| Defibrillation electrodes | Adult/Pediatric | Gel-based |
| Conductive gel | ≥ 250 ml | High conductivity |
| Manual resuscitator (Ambu bag) | Adult/Pediatric/Neonate | With oxygen reservoir |

2. List of Consumables for the Neurosurgical Department

| No. | Item | Sizes / Specifications | Technical Requirements |
| --- | --- | --- | --- |
| 1 | Ventricular catheters | 12–15 Fr | Radiopaque stripe |
| 2 | CSF drainage systems | Closed | Graduated scale |
| 3 | Subdural drains | 10–12 Fr | Silicone |
| 4 | Hydrocolloid dressings | 5×5, 10×10, 15×15 cm | High absorbency |
| 5 | Silicone mesh dressings | Various | Atraumatic removal |
| 6 | Lumbar puncture kits | Quincke 18–22G | Manometer, collection tubes |
| 7 | Tuohy needles | 16–18G | For CSF drainage |
| 8 | Skin hemostatic clips | Standard | Stainless steel |
| 9 | Neurosurgical cottonoids | Various sizes | High absorbency |

3. List of Consumables for the Gynecology Department

| No. | Item | Sizes / Specifications | Technical Requirements |
| --- | --- | --- | --- |
| 1 | Disposable trocars | 5, 10, 12 mm | Valve system, atraumatic tip |
| 2 | Veress needle (safety) | 120–150 mm | Automatic tip protection |
| 3 | Laparoscopic electrodes | Hook, needle, ball | Compatible with ESU |
| 4 | Laparoscopic clips | Small / Medium / Large | Titanium, disposable |
| 5 | Clip appliers | 5–10 mm | Clip-compatible |
| 6 | Endoscopic specimen bags | S / M / L | Durable, disposable |
| 7 | Resectoscopic loops | 90°, 120° | Disposable, sterile |
| 8 | Hysteroscopic electrodes | Ball, needle | Resectoscope compatible |
| 9 | Cervical cannulas | Various | Atraumatic |
| 10 | Fluid delivery systems | Standard | Pressure and volume control |
| 11 | Irrigation solution bags | 3–5 L | Pump compatible |
| 12 | TVT/TOT sling systems | Standard | Synthetic, sterile |
| 13 | Sterilization rings | Standard | Medical-grade plastic/silicone |
| 14 | Uterine manipulators | Disposable | Universal, with channel |
| 15 | Vaginal specula | Sizes 1–5 | Disposable, sterile |
| 16 | Vaginal tampons | Standard | High absorbency |
| 17 | Hydrocolloid dressings | 5×5, 10×10 cm | Postoperative wound protection |
| 18 | Silicone dressings | Various | Atraumatic |
| 19 | Gauze swabs | 5×5, 10×10 cm | Sterile |
| 20 | Endometrial biopsy cannulas | Various | Flexible, disposable |
| 21 | Cytology brushes | Standard | Cervical cytology sampling |
| 22 | Uterine sounds | Various | Graduated |
| 23 | Vacuum aspiration cannulas | Various | Disposable |

4. List of Consumables for the Urology Department

| No. | Item | Sizes / Specifications | Technical Requirements |
| --- | --- | --- | --- |
| 1 | Double-J ureteral stents | 4–8 Fr, 22–30 cm | Silicone/polyurethane, radiopaque |
| 2 | Urological guidewires | 0.025″–0.038″ | Hydrophilic, soft/stiff tip |
| 3 | Urological introducers | 6–12 Fr | Valve mechanism |
| 4 | Stone retrieval baskets (Dormia) | 1.5–2.2 mm | Nitinol, 3–4 wire |
| 5 | Dilation balloons | 4–10 mm | High-pressure |
| 6 | Nephrostomy catheters | 8–16 Fr | Locking pigtail |
| 7 | Ureteral catheters | 4–7 Fr | Flexible, atraumatic |
| 8 | Percutaneous access kits | Standard | US/fluoroscopy guided |
| 9 | TUR resection loops | Various | Mono/bipolar |
| 10 | TUR electrodes | Knife, loop | Resectoscope compatible |
| 11 | Resectoscope obturators | Standard | Compatible |
| 12 | Irrigation tubing | Standard | High flow |
| 13 | Irrigation bags | 3–5 L | Disposable |
| 14 | Urological meshes | Various | Polypropylene |
| 15 | Stress incontinence slings | Standard | Synthetic |
| 16 | Bladder neck balloons | Various | Temporary dilation |
| 17 | Cystoscopy kits | Disposable | Sterile |
| 18 | Urodynamic catheters | Various | Radiopaque |
| 19 | Biopsy forceps | Standard | Disposable |
| 20 | Specimen containers | Standard | Leak-proof |
| 21 | Enteral feeding syringes (Janet) | 100–150 ml | High-resistance plunger |

5. List of Consumables for the Department of General Surgery

| No. | Item | Sizes / Specifications | Technical Requirements |
| --- | --- | --- | --- |
| 1 | Surgical blades | No. 10–24 | Disposable |
| 2 | Radiopaque gauze sponges | 10×10, 30×30 cm | Mandatory radiopaque thread |
| 3 | Linear staplers | 30 / 45 / 60 mm | Compatible with cartridges |
| 4 | Circular staplers | 21 / 25 / 28 / 31 mm | Gastrointestinal surgery |
| 5 | Stapler cartridges | 2.5 / 3.5 / 4.8 mm | Various tissue thickness |
| 6 | Clip appliers | Standard | Disposable |
| 7 | Surgical clips | Small / Medium / Large | Titanium/polymer |
| 8 | Trocars | 5 / 10 / 12 mm | Valve system |
| 9 | Veress needle | 120–150 mm | Safety tip |
| 10 | Insufflation tubing | Standard | CO₂ compatible |
| 11 | Laparoscopic electrodes | Hook, spatula, ball | ESU compatible |
| 12 | Endoscopic retrieval bags | S / M / L | Specimen extraction |
| 13 | Hernia meshes | 6×11, 10×15, 15×20 cm | Polypropylene, macroporous |
| 14 | Intraperitoneal meshes | 20×30 cm | Anti-adhesive coating |
| 15 | Mesh fixation devices | Spiral / clips | Applicator compatible |

6. List of Consumables for Trauma and Orthopedic Departments

| No. | Item | Sizes | Technical Requirements |
| --- | --- | --- | --- |
| 1 | Arthroscopic cannulas | 4.0–5.5 mm | Disposable |
| 2 | Shaver blades | Various | Motor compatible |
| 3 | Tendon sutures | No. 2–5 | High tensile strength |
| 4 | Irrigation pump tubing | Lines | High flow |
| 5 | Sterile dressings | 5×5, 10×10, 15×15 cm | Hypoallergenic |
| 6 | Silver dressings | Various | Antibacterial |
| 7 | Silicone dressings | Various | Atraumatic |
| 8 | Elastic bandages | 5–10 cm | Compression |
| 9 | Plaster bandages | 5–15 cm | Fast setting |
| 10 | Synthetic casting tapes | 5–12.5 cm | Water-resistant |
| 11 | Gauze swabs | 5×5, 10×10 cm | Sterile |
| 12 | Immobilization splints | Adult/Pediatric | Aluminum/plastic |
| 13 | Vacuum splints | Various | Reusable |
| 14 | Fixation bandages | Various | Elastic |
| 15 | Orthopedic braces | S–XL | Adjustable |

7. List of Consumables for ENT Departments

| No. | Item | Sizes / Specifications | Technical Requirements |
| --- | --- | --- | --- |
| 1 | Anterior nasal packs | S / M / L | Hemostatic, sterile |
| 2 | Posterior nasal packs | Various | With fixation string |
| 3 | Hemostatic sponges | Standard | Resorbable |
| 4 | Nasal septal splints | Various | With airway channels |
| 5 | Septal plates | Standard | Silicone/polymer |
| 6 | Suction cannulas | 2–4 mm | Suction compatible |
| 7 | Endonasal catheters | Various | Atraumatic |
| 8 | Endoscopic specimen bags | S / M | Disposable |
| 9 | Ventilation tubes (grommets) | 0.9–1.5 mm | Silicone/teflon |
| 10 | Ear wicks | Various | Atraumatic |
| 11 | Myringotomy blades | Standard | Disposable |
| 12 | Silicone drains | Small | Middle ear use |
| 13 | Hemostatic materials | Standard | Microsurgical use |
| 14 | Laryngeal probes | Various | Atraumatic |
| 15 | Tracheostomy tubes | No. 4–10 | Cuffed / uncuffed |
| 16 | Tracheostomy tube holders | Standard | Adjustable |
| 17 | Suction catheters | 6–14 Fr | Sterile |
| 18 | Sinus irrigation catheters | Various | Atraumatic |
| 19 | Drug administration cannulas | Various | Flexible |
| 20 | Specimen containers | Standard | Leak-proof |

8. List of Consumables for Plastic (Reconstructive and Aesthetic) Surgery

| No. | Item | Sizes | Technical Characteristics |
| --- | --- | --- | --- |
| 1 | Disposable liposuction cannulas | 2–6 mm | Various tip designs |
| 2 | Lipofilling cannulas | 1.5–3.0 mm | Atraumatic |
| 3 | Aspiration tubing | 3–5 m | Silicone |
| 4 | Fat collection containers | Standard | Sterile |
| 5 | AFT systems (fat filtration) | Standard | Closed system |
| 6 | Lipofilling syringes | 10–60 ml | Luer-lock |
| 7 | Microvascular clips | Various | Titanium |
| 8 | Vascular suture material | 6/0–9/0 | Polypropylene |
| 9 | Non-absorbable sutures | 3/0–6/0 | Polypropylene, nylon |
| 10 | Absorbable sutures | 3/0–5/0 | Vicryl-type |
| 11 | Skin staples | Standard | Sterile |
| 12 | Staple applicators | Standard | Disposable |
| 13 | Skin adhesive | 0.5–1.0 ml | Cyanoacrylate |
| 14 | Compression dressings | Various | Postoperative support |
| 15 | Compression garments | XS–XL | Post-mammoplasty/liposuction |

9. Drainage Systems

| No. | Item | Sizes | Technical Requirements |
| --- | --- | --- | --- |
| 1 | Silicone drains | 10–24 Fr | Flexible |
| 2 | Redon / Hemovac drains | 200 / 400 ml | Active suction |
| 3 | Passive drains | Various | PVC / silicone |
| 4 | JP / Redon drains | 100 / 200 / 400 ml | Closed system |

10. Infusion and Injection Consumables

| No. | Item | Sizes | Technical Requirements |
| --- | --- | --- | --- |
| 1 | Peripheral IV catheters | 14–24G | Radiopaque |
| 2 | Infusion sets | Standard | Luer-lock |
| 3 | Three-way stopcocks | Standard | Pressure ≥ 300 psi |
| 4 | Syringes | 2–60 ml | Luer-slip / Luer-lock |
| 5 | Injection needles | 18–25G | Silicone-coated |
| 6 | Peripheral IV catheters | 14–24G | With valve, radiopaque |
| 7 | Central venous catheters | 1–3 lumen | J-tip guidewire |
| 8 | Infusion sets with filter | Standard | Filter 15–18 µm |

11. Catheterization and Patient Care

| No. | Item | Sizes | Technical Characteristics |
| --- | --- | --- | --- |
| 1 | Foley catheters | 12–22 Fr | Balloon 5–30 ml |
| 2 | Urine drainage bags | 1.5–2 L | Anti-reflux |
| 3 | Nasogastric tubes | 10–18 Fr | Radiopaque |
| 4 | Enteral feeding syringes (Janet) | 100–150 ml | Reinforced plunger |
| 5 | Underpads | 60×90 cm | Absorbent |
| 6 | Ostomy bags | Standard | Disposable |
| 7 | Pressure ulcer prevention pads | Standard | Disposable |
| 8 | Medical tapes and fixators | Various | Hypoallergenic |
| 9 | Suction tubing | 10–18 Fr | Radiopaque |

12. Infection Control Supplies

| No. | Item | Specifications |
| --- | --- | --- |
| 1 | Sterile gloves | Latex/nitrile, S–XL |
| 2 | Non-sterile gloves | Latex/nitrile, S–XL |
| 3 | Medical masks | 3-ply |
| 4 | Disposable gowns | 25–40 g/m² |
| 5 | Caps and shoe covers | Nonwoven material |
| 6 | Medical waste bags | Classes A, B, C |

13. Laboratory Consumables

| No. | Item | Characteristics |
| --- | --- | --- |
| 1 | Blood collection tubes | EDTA / Serum |
| 2 | Specimen containers | Leak-proof |
| 3 | Microscope slides | Standard |
| 4 | Blood collection tubes | EDTA / Serum |
| 5 | Biopsy containers | With formalin |
| 6 | Cytology fixatives | Aerosol / solution |